Experiment 9: Queue (Producer-Consumer)

A hands-on exploration of inter-task communication using FreeRTOS queues on STM32 microcontrollers. This

experiment demonstrates how tasks safely exchange data in a real-time embedded environment.

FREERTOS LAB

STM32

EMBEDDED SYSTEMS

Aim

This experiment is designed to build a solid understanding of how tasks communicate in a real-time operating system

environment. By the end of this lab, you will have practical experience with one of FreeRTOS's most important IPC

mechanisms.

Inter-Task Communication

Implement a reliable mechanism for two independent

FreeRTOS tasks to exchange data without direct

memory access or shared variable conflicts. The

producer task will generate sensor readings while the

consumer task processes them, both running

concurrently under the FreeRTOS scheduler.

FreeRTOS Queue Usage

Learn to create, initialise, and operate a FreeRTOS

queue as a thread-safe buffer for passing integer data

between a producer task and a consumer task. You

will configure queue depth, item size, and handle

blocking behaviour during send and receive

operations.

This experiment uses the CMSIS-RTOS2 API layer (

osMessageQueue

functions) which is portable across RTOS

implementations.

Apparatus

The following hardware and software components are required to complete this experiment. Ensure all connections are

verified before flashing the firmware to the board.

STM32 Board

STM32F4 or STM32F7 series

Nucleo/Discovery board running

FreeRTOS. The board provides

sufficient RAM and processing

capability to run multiple

concurrent tasks with a queue IPC

mechanism.

Ultrasonic Sensor

HC-SR04 module used by the

producer task to generate real-

time distance measurement data.

The sensor triggers a pulse and

measures echo return time to

calculate distance in centimetres.

STM32CubeIDE

Integrated development

environment for project

configuration, FreeRTOS

middleware setup, code writing,

and on-chip debugging. CubeMX

is used to configure the RTOS and

generate initialisation code.

Theory: FreeRTOS Queues

A

queue

in FreeRTOS is a

FIFO (First In, First Out)

data structure

that allows tasks and interrupt service routines to safely pass

messages. It is the primary inter-task communication (IPC)

mechanism provided by FreeRTOS.

Queues are

thread-safe

by design — the kernel handles all

synchronisation internally, eliminating race conditions when

multiple tasks access shared data. Each queue item is copied by

value, not by reference.

FIFO Order

Items are read in the same

order they were written.

Fixed Item Size

All items in a queue must

be the same data type and

size.

Blocking Support

Tasks can block while waiting to send or receive.

Queue Properties

Queue length (depth)

Item size in bytes

Blocking timeout (ticks)

Copy-by-value semantics

ISR-safe API variants

Concept: Producer-Consumer Model

The producer-consumer pattern is a classic concurrency design where one task generates data and another task

processes it. The queue acts as a

bounded buffer

between them, decoupling the two tasks in time and execution speed.

Consumer Task

FreeRTOS Queue

Producer Task

The diagram above illustrates the data flow: the producer pushes measurements into the queue, the queue holds them

in FIFO order, and the consumer retrieves and processes each value independently.

Producer Task

Reads the ultrasonic sensor at regular intervals and

pushes the distance value into the queue using

osMessageQueuePut()

. If the queue is full, the producer

can block or drop the sample based on the timeout

parameter.

Consumer Task

Waits on the queue using

osMessageQueueGet()

. When

data is available, it reads the value and processes it — for

example, printing to UART or triggering an alert if the

distance falls below a threshold.

Working Principle

The experiment operates as a continuous pipeline: sensor data flows from acquisition to processing through the queue.

Understanding the timing behaviour of each stage is key to tuning system performance.

01

Sensor Task Generates Data

The producer task triggers the HC-SR04 ultrasonic sensor,

measures the echo pulse width, and converts it to a

distance value in centimetres. This integer value is

prepared for queue insertion.

02

Queue Stores Values

The distance value is copied into the queue's internal

buffer. The queue maintains FIFO order and tracks the

count of available items. If the queue is full, the producer

blocks for the specified timeout.

03

Consumer Reads Values

The consumer task unblocks when data is available,

retrieves the oldest value from the queue, and processes it.

If the queue is empty, the consumer blocks until new data

arrives or the timeout expires.

04

Repeat Continuously

Both tasks run in an infinite loop under the FreeRTOS

scheduler, which switches between them based on priority

and blocking state, ensuring continuous data flow.

Implementation Details

Task Configuration

Producer Task:

Stack 128 words, Priority Low

Consumer Task:

Stack 128 words, Priority Low

Both tasks run in infinite

while(1)

loops

Scheduler starts after queue creation

Queue Configuration

Data Type:

uint32_t

(integer)

Queue Depth:

5 items (configurable)

Item Size:

sizeof(uint32_t)

Created before

osKernelStart()

Key Rule:

Always create the queue

b

e

f

o

r

e

starting the scheduler. The queue handle must be accessible to both

tasks — typically declared as a global or passed via task parameters.

In STM32CubeIDE, enable FreeRTOS middleware in the project settings, then use CubeMX to configure two tasks and a

queue object. The generated code provides handle variables that you will use in your application logic.

Code Logic

The CMSIS-RTOS2 API provides two core functions for queue operations. These functions handle all synchronisation

internally and support optional blocking timeouts.

osMessageQueuePut()

Sender (Producer)

osMessageQueuePut(

queueHandle,

&

data

,

// pointer to item

0

,

// priority

100

// timeout ticks

;

)

Returns

osOK

on success. Blocks if queue is full.

osMessageQueueGet()

Receiver (Consumer)

osMessageQueueGet(

queueHandle,

&data,

// pointer to buffer

NULL

,

// priority

100

// timeout ticks

)

;

Returns

osOK

on success. Blocks if queue is empty.

Delay Strategy

Both tasks use

osDelay(1000)

for a

1-

second interval

between operations. This creates a balanced producer-

consumer rate. Adjusting delays demonstrates queue

fill/empty behaviour.

Timeout Parameter

A timeout of

100

ticks means the task waits up to 100 ms

for the operation to complete. Use

osWaitForever

for

indefinite blocking.

Observations

By varying task priorities and delay values, several important real-time system behaviours can be observed during this

experiment.

Producer Faster → Queue

Fills

When the producer delay is shorter

than the consumer delay, the queue

gradually fills. Once full,

osMessageQueuePut()

returns an error

or blocks, demonstrating back-

pressure.

Consumer Slower → Data

Delay

A slower consumer introduces latency

between data generation and

processing. Older readings may

become stale before they are

consumed — a critical consideration

in real-time systems.

Priority Affects Execution

Increasing the consumer's priority

ensures it runs before the producer

when both are ready, reducing queue

occupancy. Priority inversion

scenarios can also be explored.

1000

ms

Default Delay

Interval between each queue

operation in balanced mode.

5

Queue Depth

Maximum items buffered before

producer blocks.

2

Active Tasks

Producer and Consumer running

concurrently under scheduler.

Conclusion

This experiment successfully demonstrates the power and simplicity of FreeRTOS queues as a thread-safe inter-task

communication mechanism. The producer-consumer model implemented here is foundational to many real-world

embedded applications.

✅

Safe Communication

Queues eliminate race conditions

by handling all synchronisation

internally. Tasks never access

shared memory directly.

✅

Data Integrity

Copy-by-value semantics ensure

data is not corrupted by

concurrent access. Each item is

atomically transferred.

✅

Scalable Pattern

The producer-consumer model

scales to multiple producers,

multiple consumers, and can be

extended with multiple queues for

different data types.

FreeRTOS queues provide a robust, kernel-managed solution for inter-task data exchange — a fundamental building

block for any real-time embedded application.

EXPERIMENT COMPLETE

NEXT: SEMAPHORES & MUTEXES