EMBEDDED SYSTEMS LAB

Experiment 7: FreeRTOS Task Priority

Explore how FreeRTOS schedules tasks on the STM32 platform using priority-based preemption. This experiment

demonstrates real-time task scheduling, LED control, and the impact of priority assignment on CPU execution.

STM32 + FREERTOS

TASK SCHEDULING

PRIORITY ANALYSIS

OBJECTIVE

Aim of the Experiment

This experiment is designed to give hands-on experience with FreeRTOS task management on the STM32

microcontroller. By the end, you will understand how the RTOS scheduler allocates CPU time based on task priorities

and observe the practical effects on hardware output.

1

Create Two FreeRTOS

Tasks

Define and register two

independent tasks within the

FreeRTOS kernel, each

responsible for controlling a

separate LED on the Nucleo

board.

2

Assign Different

Priorities

Configure each task with a

distinct priority level to observe

how the scheduler decides which

task gets CPU time when both are

ready to run.

3

Analyse LED Behaviour

Observe and record the blinking

patterns of both LEDs under equal

and unequal priority

configurations to validate

theoretical scheduling behaviour.

SETUP

Apparatus Required

Gather the following hardware and software components before beginning the experiment. Ensure all connections are

secure and the IDE is properly configured for your board.

STM32 Nucleo-F446RE

The main development board

featuring the ARM Cortex-M4F core.

This board supports FreeRTOS and

provides onboard debugging via ST-

LINK.

LEDs & Breadboard

Two LEDs with current-limiting

resistors (220

Ω

–330

Ω

connected to

)

GPIO pins for visual output. A

breadboard simplifies wiring and

testing.

STM32CubeIDE & CubeMX

ST's official IDE for project creation,

code generation, and debugging.

CubeMX handles peripheral and

FreeRTOS configuration through a

graphical interface.

THEORY

Priority-Based Scheduling in FreeRTOS

FreeRTOS implements a

preemptive, priority-based scheduler

. Each task is assigned a numeric priority — the higher

the value, the more urgently the scheduler treats that task. When a higher-priority task becomes ready to run, it

immediately preempts any lower-priority task currently executing.

High Priority Task

Executes first whenever it is in the Ready state.

Preempts lower-priority tasks without waiting for them

to yield or block.

Runs immediately on unblocking

Preempts lower-priority tasks

Gets CPU time preferentially

Low Priority Task

Executes only when no higher-priority task is ready. May

experience delayed execution or even starvation if a

high-priority task never blocks.

Runs when higher tasks are blocked

Delayed if high-priority task is ready

May starve under constant preemption

CORE CONCEPT

Preemptive Scheduling & CPU Sharing

In a preemptive RTOS, the kernel controls the CPU and decides at every moment which task should run. Priority is the

primary factor in this decision. Understanding this mechanism is essential for designing reliable real-time systems.

Key Insight:

A task only gives up the CPU when it

blocks

(e.g., via

osDelay()

)

,

yields

, or is

preempted

by a higher-

priority task becoming ready.

EXPERIMENT DESIGN

How the Experiment Works

Two FreeRTOS tasks are created —

LED_1

and

LED_2

— each toggling a separate LED with the same delay interval. The

scheduler determines which task runs based on their assigned priorities. By changing priority values, we observe

distinct behavioural patterns.

1

Task Creation

LED_1 and LED_2 tasks are defined

with separate function bodies and

registered with

osThreadNew()

.

2

Priority Assignment

Each task receives a priority value

(

e.g.,

osPriorityNormal

or

osPriorityHigh

)

during creation.

3

Scheduler Controls

Execution

FreeRTOS scheduler runs the

highest-priority ready task. Both

tasks call

osDelay(500)

to block and

yield CPU.

IMPLEMENTATION

Step-by-Step Implementation

Follow these steps in STM32CubeIDE to set up the experiment. CubeMX handles most of the configuration, letting you

focus on task logic.

01

Configure GPIO Pins

In CubeMX, set two GPIO pins (e.g., PA5 and PB10) as

GPIO_Output

for LED_1 and LED_2. Enable the

corresponding ports in the pinout view.

02

Enable FreeRTOS

Navigate to

Middleware

→

FreeRTOS

in CubeMX. Select

the CMSIS_V2 API, set heap size (e.g.,

configTOTAL_HEAP_SIZE

= 12KB), and enable the scheduler.

03

Create Two Tasks

In the FreeRTOS configuration, add two tasks:

LED_1_Task

and

LED_2_Task

. Assign stack sizes (e.g., 128 words) and

set initial priorities. Generate code.

04

Add LED Toggle Logic

In each task function, add

HAL_GPIO_TogglePin()

followed by

osDelay(500)

. Build and flash to the Nucleo board.

CODE LOGIC

Inside Each Task Function

Each task follows a simple infinite loop: toggle the LED, send a debug message via SWV, then delay for 500 ms. The delay

is critical — it blocks the task, allowing the scheduler to switch to the other task.

LED_1 Task

void

LED_1_Task

void *arg)

(

{

for (;;) {

HAL_GPIO_TogglePin

(

GPIOA, GPIO_PIN_5);

ITM_SendChar

(

'

1

');

osDelay

(

500

)

;

}

}

LED_2 Task

void

LED_2_Task

(

void *arg)

{

for (;;) {

HAL_GPIO_TogglePin

(

GPIOB, GPIO_PIN_10);

ITM_SendChar

(

'

2

');

osDelay

(

500

)

;

}

}

Both tasks use

osDelay(500)

to block for 500 ms, yielding

CPU time. The SWV output via

ITM_SendChar()

lets you

trace execution in the IDE's Live Watch.

Tip:

Use the

SWV Trace

feature in STM32CubeIDE to monitor task execution in real time without halting the

processor.

RESULTS

Observations Across Priority Configurations

Run the experiment with three different priority configurations and record the LED behaviour. The table below

summarises expected outcomes for each scenario.

Configuration

LED_1 Behaviour

LED_2 Behaviour

Equal Priority (Normal /

Normal)

Blinks regularly every 500 ms

Blinks regularly every 500 ms — both

share CPU equally

LED_1 Higher (High /

Normal)

Blinks on time, dominates CPU

Blinks slower or irregularly — gets CPU

only when LED_1 delays

LED_1 Much Higher (no

delay)

Continuous toggling, no blocking

Starved — LED_2 may never execute if

LED_1 never yields

ms

500

Delay Interval

Both tasks block for 500 ms per cycle,

allowing the scheduler to switch

context.

2

Tasks Created

LED_1 and LED_2 run concurrently

under FreeRTOS kernel management.

5

Priority Levels

FreeRTOS supports multiple priority

levels — osPriorityIdle to

osPriorityRealtime.

SUMMARY

Conclusion & Key Takeaways

This experiment demonstrates the fundamental behaviour of FreeRTOS priority-based scheduling on real hardware. The

LED observations directly reflect how the kernel allocates CPU time — a critical concept for any embedded real-time

system designer.

Priority Dictates

Execution

Higher-priority tasks always

execute before lower-priority

ones. This is deterministic and

predictable — a core RTOS

guarantee.

High Priority Dominates

CPU

A task that never blocks will starve

all lower-priority tasks. Always

ensure high-priority tasks yield or

block periodically.

RTOS Scheduling is

Visible

Physical LED output and SWV

trace make abstract scheduling

concepts tangible. Use both for

debugging and validation.

Understanding task priority and preemption is the foundation of reliable real-time embedded software. Always

design with scheduling behaviour in mind from the start.