PWM Signal Generation using STM32F446RE

RTOS Lab Experiment 4

Introduction to PWM

Pulse Width Modulation (PWM) is a powerful technique used to control analog devices using digital signals. This

method allows microcontrollers to produce variable power levels without requiring complex analog circuitry, making it

ideal for embedded systems applications.

In PWM, a digital signal rapidly switches between HIGH and LOW states, effectively creating an analog-like output. The

technique is widely employed in embedded systems for controlling LED brightness, motor speed, servo positions, and

power delivery efficiently.

One of the key advantages of PWM over traditional analog control methods is its ability to reduce power loss

significantly. Since the switching device is either fully ON or fully OFF, minimal power is dissipated as heat during

operation, leading to improved efficiency and reduced component stress.

Experiment Objectives

Understand PWM Generation

Learn the fundamental concepts behind PWM signal

generation and its applications in embedded systems

Configure STM32 Timer

Set up timer peripherals in PWM mode for precise

signal generation

Control LED Brightness

Implement duty cycle variation to control LED

intensity

Observe Real-Time Output

Monitor and verify PWM output variations in real-

time

Components Used

This experiment requires the following hardware and software components:

STM32 Nucleo-F446RE

Development board featuring STM32F446RE

microcontroller with integrated debugging

USB Cable

Standard USB Type-A to Micro-B cable for power

and programming

STM32CubeIDE

Integrated development environment for writing,

building, and debugging code

STM32CubeMX

Graphical configuration tool for initializing

peripherals and generating initialization code

PWM Concept Explained

What is PWM?

PWM signal is a square wave that

alternates between HIGH and

LOW states at a fixed frequency.

The key parameter is the

duty

cycle

, which represents the

percentage of time the signal

remains in the HIGH state during

each period.

How Duty Cycle Works

The duty cycle directly controls

the average voltage delivered to

the load. By rapidly switching the

signal, the effective power can be

precisely controlled without

changing the supply voltage.

0

% Duty Cycle

LED remains completely OFF as the signal never goes HIGH

50

% Duty Cycle

LED shows medium brightness with equal ON and OFF periods

100

% Duty Cycle

LED reaches maximum brightness as the signal stays HIGH

continuously

Working Principle

The STM32 timer generates PWM signals using an internal counter that continuously increments and compares its

value with a predefined threshold.

Internal Counter

Timer counter increments

continuously at a configured

frequency

Comparison

Counter value is compared with

Capture Compare Register (CCR)

Output Generation

Output goes HIGH when counter <

CCR, LOW when counter

≥

CCR

By changing the CCR value, we can modify the duty cycle, which in turn changes the LED brightness. This comparison

happens continuously, creating a stable PWM signal.

Timer Configuration

PWM is generated using TIM2 timer peripheral on the STM32F446RE microcontroller. Proper configuration of timer

parameters is essential for stable output generation.

Prescaler (PSC)

Controls clock division by dividing

the timer clock frequency.

Determines how fast the counter

increments. Lower values

produce higher frequency PWM.

Auto Reload Register (ARR)

Defines the period of the PWM

signal by setting the maximum

counter value. Determines PWM

frequency along with prescaler.

Capture Compare Register

(

CCR

)

Controls duty cycle by setting the

comparison threshold. Changing

this value during runtime adjusts

LED brightness.

Configuration Tip:

The relationship between these parameters determines the PWM frequency and

resolution. For LED control, a frequency above 100 Hz is recommended to avoid visible flickering.

Step-by-Step Procedure

01

Open STM32CubeMX

Launch the graphical configuration tool

02

Select STM32F446RE

Choose the correct microcontroller from the device list

03

Configure PA5 as TIM2 Channel

Set pin PA5 as PWM output using TIM2 channel 1

04

Set PSC and ARR Values

Configure prescaler and auto-reload for desired frequency

05

Generate Code

Click "Generate Code" to create initialization files

06

Open in STM32CubeIDE

Import the generated project into the IDE

07

Write PWM Logic

Implement duty cycle variation in main loop

08

Build and Run

Compile code and flash to the board

Results and Observations

The LED brightness varies smoothly as the duty cycle changes, demonstrating successful PWM implementation. The

fade-in and fade-out effect is clearly visible when cycling through different duty cycle values.

Low Duty Cycle

LED appears dim or barely visible

as the signal spends most time in

OFF state. Average power

delivered is minimal.

Medium Duty Cycle

LED shows moderate brightness

with noticeable glow. Balanced

ON and OFF periods provide

intermediate intensity.

High Duty Cycle

LED reaches maximum

brightness approaching full

intensity. Signal remains HIGH

for most of the period.

Key Observation:

The transition between different brightness levels is smooth and continuous, confirming proper

PWM signal generation. This validates the relationship between duty cycle percentage and average output voltage.

Conclusion

PWM provides an efficient method for controlling analog output using digital systems. This experiment successfully

demonstrates the fundamental relationship between duty cycle and output intensity, forming a solid foundation for

advanced embedded applications.

The implementation on STM32F446RE showcases the practical application of timer peripherals for signal generation.

By understanding and mastering PWM techniques, engineers can develop sophisticated control systems for motor

speed regulation, LED lighting, power management, and automation applications.

Technical Achievement

Successful PWM signal

generation using STM32 timer

peripherals

Learning Foundation

Understanding core concepts

applicable to motor control and

automation

Practical Skills

Hands-on experience with

STM32CubeMX configuration and

real-time control

This experiment demonstrates a crucial embedded systems technique with wide-ranging applications in

modern electronics and industrial control systems.